SIMATS ENGINEERING

ASSESSMENT TOOL 1: Analytical Problem Solving

COURSE NAME: COMPUTER VISION		            COURSE CODE:	ITA05

COURSE OUTCOME COVERED

CO2: Apply image enhancement and segmentation techniques for extracting meaningful regions from images. (BTL 3)

Assessment Tool Weightage: 50%

1. Grey Level Transformation

(a) Explain the purpose of grey level transformation in image enhancement.
(b) Apply a negative transformation s=255-r to the pixel values:
r=[50,100,150,200]

Compute the transformed values.

2. Log Transformation

(a) Explain how log transformation enhances low-intensity pixels.
(b) Given s=clog⁡(1+r), where c=10, compute s for:
r=[0,10,50,100]

3. Histogram Processing

(a) Explain the role of histogram equalization in contrast enhancement.
(b) Given a 3-bit image with histogram:

| Intensity | Frequency |
| --- | --- |
| 0 | 2 |
| 1 | 2 |
| 2 | 4 |
| 3 | 8 |

Compute the cumulative distribution and equalized intensity values.

4. Spatial Filtering (Smoothing)

(a) Explain the purpose of spatial filtering for smoothing.
(b) Apply a 3×3 averaging filter to the following region:

10 20 30 20 30 40 30 40 50

Compute the new center pixel value.

5. Spatial Filtering (Sharpening)

(a) Explain how sharpening enhances image details.
(b) Apply the Laplacian mask:

[0 -1       0}, {-1       4 -1],   { 0 -1      0 ]

to the matrix:

10   10   10, 10   20   10, 10   10   10

Compute the output at the center.

6. Frequency Domain Filtering

(a) Differentiate low-pass and high-pass filtering in frequency domain.
(b) Given frequency components:
F=[100,80,40,20]

Apply a low-pass filter that retains only the first two components. Write the filtered output.

7. Thresholding

(a) Explain global thresholding in segmentation.
(b) Given pixel values:
200

Apply threshold T=100. Assign values (0 or 1).

8. Edge Detection

(a) Explain the concept of edge detection.
(b) Apply the Sobel operator (horizontal):

-1 -2 -1,  0    0    0,   1   2   1

to:

10   10   10,   20   20   20,  30   30   30

Compute the gradient at the center.

9. Region-Based Segmentation

(a) Explain region growing in segmentation.
(b) Given pixel intensities:
1   0   2

If seed = 50 and threshold difference = 5, identify pixels belonging to the region.

10. Combined Enhancement & Segmentation

(a) Explain why smoothing is applied before segmentation.
(b) Given image values:
1   0   5

First apply averaging (assume mean = 48), then threshold at T=50.
Determine final segmented output.

11. Power-Law (Gamma) Transformation

A grayscale image is enhanced using power-law transformation.
(a) Interpret the role of gamma correction in image enhancement.
(b) Justify the use of transformation s=crγ, where γ=0.5, c=1.
(c) Apply the transformation step-by-step to pixel values:
r=[25,  100,  225].
(d) Compute the transformed values accurately.
(e) Interpret the results in terms of brightness enhancement.

12. Contrast Stretching

An image has intensity values in the range [50, 150].
(a) Interpret the need for contrast stretching.
(b) Justify linear transformation to range [0, 255].
(c) Apply transformation step-by-step for values:
r=[50,100,150].
(d) Compute the output values accurately.
(e) Interpret improvement in contrast.

13. Histogram Equalization (4-bit Image)

A 4-bit image has the following histogram:

| Intensity | Frequency |
| --- | --- |
| 0 | 4 |
| 1 | 6 |
| 2 | 10 |
| 3 | 6 |

(a) Interpret the need for equalization.
(b) Justify use of CDF.
(c) Compute cumulative distribution.
(d) Compute new intensity levels.
(e) Interpret contrast enhancement.

14. Median Filtering

A noisy image contains salt-and-pepper noise.
(a) Interpret noise characteristics.
(b) Justify median filter usage.
(c) Apply filter to:
10    200    10, 200    50    200, 10    200    10

(d) Compute center value.
(e) Interpret noise removal effectiveness.

15. Gaussian Smoothing

An image is smoothed using Gaussian filter.
(a) Interpret smoothing objective.
(b) Justify Gaussian filter over averaging.
(c) Apply filter:
[1    2    1,   2    4    2,   1    2    1]

to matrix:
[10    20    10, 20    40    20, 10    20    10]

(d) Compute center pixel.
(e) Interpret smoothing effect.

16. High-Pass Filtering

An image requires sharpening.
(a) Interpret need for high-pass filtering.
(b) Justify filter selection.
(c) Apply mask:
[-1   -1   -1, -1    8  -1, -1   -1   -1]

(d) Compute center value.
(e) Interpret edge enhancement.

17. Frequency Domain High-Pass

Given frequency components:
F=[10,20,80,100]

(a) Interpret high-frequency importance.
(b) Justify high-pass filtering.
(c) Apply filtering (remove low frequencies).
(d) Compute output.
(e) Interpret result.

18. Adaptive Thresholding

Pixel values: [60, 80, 120, 140]
(a) Interpret segmentation challenge.
(b) Justify adaptive thresholding.
(c) Apply local threshold (mean = 100).
(d) Compute binary output.
(e) Interpret result.

19. Edge Detection (Vertical Sobel)

Apply vertical Sobel:
[-1    0   1, -2    0    2, -1    0    1]

Matrix:
[10   20   30, 10    20    30, 10    20    30]

(a) Interpret edge detection objective.
(b) Justify operator choice.
(c) Apply step-by-step.
(d) Compute center value.
(e) Interpret edge direction.

20. Prewitt Edge Detection

(a) Interpret Prewitt operator purpose.
(b) Justify use over Sobel.
(c) Apply kernel:
[-1   0   1, -1    0    1, -1    0    1]

(d) Compute output.
(e) Interpret result.

21. Region Splitting

Pixel set: [40, 45, 100, 105]
(a) Interpret segmentation problem.
(b) Justify splitting method.
(c) Apply threshold 80.
(d) Compute regions.
(e) Interpret separation.

22. Region Merging

Regions: [45, 50], [52, 55]
(a) Interpret merging criteria.
(b) Justify merging approach.
(c) Apply threshold = 5.
(d) Compute merged region.
(e) Interpret result.

23. Edge Linking

Edges detected: [1,  0,  1,  0,  1]
(a) Interpret problem of discontinuity.
(b) Justify linking method.
(c) Apply linking step.
(d) Compute result.
(e) Interpret continuity.

24. Low-Pass Filtering (Frequency)

Given F=[200,150,50,20]
(a) Interpret smoothing requirement.
(b) Justify LPF selection.
(c) Apply filter.
(d) Compute output.
(e) Interpret effect.

25. Laplacian Sharpening (Variant)

Mask:

0    -1      0,

-1      5 -1,

0   -1       0

(a) Interpret sharpening need.
(b) Justify mask.
(c) Apply to matrix.
(d) Compute center.
(e) Interpret result.

26. Histogram Matching

(a) Interpret need for matching.
(b) Justify method.
(c) Map values [10, 50, 100].
(d) Compute new values.
(e) Interpret.

27. Noise Reduction Comparison

(a) Interpret noise type.
(b) Compare averaging vs median.
(c) Apply both.
(d) Compute outputs.
(e) Interpret differences.

28. Combined Filtering

(a) Interpret need for smoothing + sharpening.
(b) Justify sequence.
(c) Apply filters stepwise.
(d) Compute output.
(e) Interpret improvement.

29. Multi-Threshold Segmentation

Pixels: [20, 60, 120, 180]
(a) Interpret multi-level segmentation.
(b) Justify thresholds (50,100).
(c) Apply segmentation.
(d) Compute classes.
(e) Interpret.

30. Boundary Detection

(a) Interpret boundary extraction.
(b) Justify method.
(c) Apply edge + linking.
(d) Compute result.
(e) Interpret boundaries.

Code of Conduct:

I ____________________ (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 (Exemplary) | Level 3 (Proficient) | Level 2 (Developing) | Level 1 (Beginning) |
| --- | --- | --- | --- | --- | --- |
| Problem Understanding | 20% | Clearly interprets problem, identifies all parameters and requirements accurately | Understands problem with minor gaps | Partial understanding; misses key elements | Misinterprets or unable to understand problem |
| Method Selection | 20% | Selects most appropriate method/formula with strong justification | Selects correct method with minor errors | Inappropriate or partially correct method | Unable to select method |
| Solution Process | 25% | Logical, systematic steps with correct approach throughout | Mostly correct steps with minor mistakes | Disorganized steps; multiple errors | No clear process |
| Accuracy of Calculation | 20% | All calculations accurate; correct final answer | Minor calculation errors | Frequent errors; incorrect final answer | No valid calculations |
| Interpretation of Results | 15% | Clearly interprets results with units and engineering relevance | Basic interpretation with minor omissions | Limited or unclear interpretation | No interpretation |