2. Automated Video Surveillance with Alert System

1. Objective

The system should:

- Detect moving objects and unusual activity.

- Generate alerts with minimum delay.

- Work with standard CCTV video.

- Reduce false alarms caused by rain, shadows, trees, lighting changes, and other dynamic backgrounds.

2. Proposed OpenCV Architecture

CCTV Video Feed

↓

Frame Capture

↓

Preprocessing

(Resize + Grayscale + Gaussian Blur)

↓

Background / Motion Detection

↓

Noise Removal

(Morphological Operations)

↓

Contour Detection

↓

Feature Detection

(ORB)

↓

Motion Analysis

(Optical Flow)

↓

Activity Verification

↓

Alert Decision

↓

Real-Time Alert + Bounding Box

3. Step-by-Step Design

Step 1: Capture CCTV Video

Use OpenCV's VideoCapture() to read frames from:

- CCTV camera

- RTSP stream

- Video file

- Webcam

cap = cv2.VideoCapture("cctv_video.mp4")

while True:

ret, frame = cap.read()

if not ret:

break

For a real CCTV deployment, the camera's RTSP stream can replace the video-file path.

Step 2: Preprocessing

Each frame is first resized and converted to grayscale.

frame = cv2.resize(frame, (640, 480))

gray = cv2.cvtColor(frame, cv2.COLOR_BGR2GRAY)

gray = cv2.GaussianBlur(gray, (5, 5), 0)

Why?

Gaussian blur reduces small image noise before motion detection.

For example:

Camera Noise

↓

Gaussian Blur

↓

Smoother Image

↓

More Reliable Motion Detection

This prevents tiny pixel changes from being incorrectly treated as movement.

4. Motion Detection

For a fixed CCTV camera, background subtraction can be used.

A practical OpenCV choice is MOG2:

bg = cv2.createBackgroundSubtractorMOG2(

history=500,

varThreshold=50,

detectShadows=True

)

mask = bg.apply(frame)

The algorithm learns the background and identifies pixels that differ significantly from it.

Background

+

Moving Object

↓

Background Subtraction

↓

Foreground Mask

5. Noise Removal

The raw foreground mask may contain small unwanted regions.

Use morphological operations:

kernel = cv2.getStructuringElement(cv2.MORPH_ELLIPSE, (5, 5))

mask = cv2.morphologyEx(mask, cv2.MORPH_OPEN, kernel)

mask = cv2.morphologyEx(mask, cv2.MORPH_CLOSE, kernel)

Purpose

- Opening removes small noise.

- Closing fills small gaps in detected objects.

This reduces false detections from small background changes.

6. Contour Detection

Find connected moving regions:

contours, _ = cv2.findContours(

mask,

cv2.RETR_EXTERNAL,

cv2.CHAIN_APPROX_SIMPLE

)

for contour in contours:

area = cv2.contourArea(contour)

if area > 500:

x, y, w, h = cv2.boundingRect(contour)

cv2.rectangle(frame, (x, y), (x+w, y+h), (0,255,0), 2)

The area threshold prevents very small changes from triggering alerts.

7. Feature Detection Using ORB

Motion detection tells us where something is moving, but feature detection can provide more information about the region.

ORB can be used:

orb = cv2.ORB_create(nfeatures=500)

keypoints, descriptors = orb.detectAndCompute(

gray, None

)

ORB is useful because it is relatively fast and suitable for real-time computer vision applications.

The detected keypoints can help characterize important visual regions.

8. Optical Flow for Motion Analysis

To understand how the detected object is moving, optical flow can be applied between consecutive frames.

For example, Lucas-Kanade optical flow:

points = cv2.goodFeaturesToTrack(

previous_gray,

maxCorners=100,

qualityLevel=0.3,

minDistance=7

)

next_points, status, error = cv2.calcOpticalFlowPyrLK(

previous_gray,

gray,

points,

None

)

The displacement of points gives information about:

- Direction

- Speed

- Movement consistency

Frame t

↓

Feature Points

↓

Frame t+1

↓

Track Feature Movement

↓

Calculate Motion

9. Unusual Activity Detection

Simply detecting motion is not enough because normal activities can also produce movement.

Therefore, use multiple conditions.

For example:

Motion detected?

↓

YES

↓

Large enough object?

↓

YES

↓

Motion persists for several frames?

↓

YES

↓

Significant motion / unusual region?

↓

YES

↓

Generate Alert

This multi-stage verification significantly reduces false positives.

Example

A tree moving because of wind:

Motion detected → Small/irregular region

↓

No Alert

A person entering a restricted area:

Motion detected → Large persistent region

↓

Consistent movement

↓

Alert

10. Handling Dynamic Backgrounds

Dynamic backgrounds are one of the biggest problems in surveillance.

Examples:

- Moving trees

- Rain

- Water

- Shadows

- Changing sunlight

- Camera noise

Solutions

1. Adaptive background model

MOG2 continuously updates its understanding of the background.

2. Area threshold

Ignore very small moving regions.

3. Temporal filtering

Require motion to persist for several consecutive frames instead of triggering an alert from one frame.

4. Morphological filtering

Remove small isolated regions and fill holes.

5. Region of Interest (ROI)

Ignore areas where movement is expected.

Example:

CCTV FRAME

┌──────────────────────────────┐

│ Trees          Sky           │ ← Ignore

│                              │

│       ┌──────────────┐       │

│       │ Restricted   │       │

│       │     Area     │       │ ← Monitor

│       └──────────────┘       │

│                              │

└──────────────────────────────┘

ROI is particularly effective for reducing false alarms.

11. Handling Noise

| Problem | Solution |
| --- | --- |
| Camera noise | Gaussian blur |
| Small foreground regions | Area threshold |
| Isolated pixels | Morphological opening |
| Holes in objects | Morphological closing |
| Temporary motion | Multi-frame confirmation |
| Shadows | MOG2 shadow detection / shadow filtering |

12. Real-Time Constraints

Real-time surveillance requires processing frames quickly.

To improve speed:

- Resize frames to 640×480 or another suitable resolution.

- Use ORB rather than computationally expensive feature methods when appropriate.

- Process only the ROI when possible.

- Avoid unnecessary processing of every pixel.

- Use efficient OpenCV functions.

- Process frames continuously and release resources properly.

The basic objective is:

rather than maximum possible accuracy at the expense of response time.

13. Alert System

When the activity satisfies the required conditions:

if motion_confirmed:

print("ALERT: Unusual activity detected!")

In a real system, this can be extended to:

- Alarm/siren

- SMS

- Email

- Mobile notification

- Security dashboard

- Event recording

The system should also save the relevant frame or short video clip as evidence.

14. Overall Evaluation

| Requirement | Proposed Solution |
| --- | --- |
| Motion detection | MOG2 background subtraction |
| Feature detection | ORB |
| Motion analysis | Optical Flow |
| Noise reduction | Gaussian Blur + Morphology |
| Dynamic background | Adaptive background model |
| False-positive reduction | ROI + area threshold + temporal confirmation |
| Real-time operation | Resizing + efficient OpenCV algorithms |
| Alert generation | Multi-condition alert logic |

Conclusion

The proposed OpenCV-based surveillance system combines background subtraction, ORB feature detection, optical flow, preprocessing, and multi-frame activity verification.

The key design decision is not to generate an alert from motion detection alone. Instead, motion is first detected, noise is removed, the moving region is analyzed, and the activity is confirmed across multiple frames. This helps distinguish genuine unusual activity from dynamic backgrounds, shadows, camera noise, and temporary motion.

Therefore, the system provides a practical balance between accuracy, low false positives, and real-time performance, making it suitable for standard CCTV-based surveillance.

5. Smart Traffic Monitoring System

1. Objective

The system should:

- Detect vehicles from a live CCTV/video stream.

- Track vehicles across consecutive frames.

- Count vehicles accurately.

- Display traffic statistics.

- Operate continuously with minimum delay.

2. Proposed OpenCV System

A simple and effective design is:

Live CCTV / Video Stream

↓

Frame Capture

↓

Preprocessing

↓

Vehicle Detection

↓

Tracking

↓

Vehicle Counting

↓

Statistics Update

↓

Visualization

↓

Live Display

For a practical system, OpenCV can handle the video processing, tracking, counting logic, and visualization. A trained object detector such as a YOLO model can be used for reliable vehicle detection.

3. Video Handling

OpenCV's VideoCapture() reads frames continuously from a camera or CCTV stream.

import cv2

cap = cv2.VideoCapture(0)

while True:

ret, frame = cap.read()

if not ret:

break

cv2.imshow("Traffic Monitoring", frame)

if cv2.waitKey(1) & 0xFF == ord('q'):

break

cap.release()

cv2.destroyAllWindows()

For CCTV, the camera's RTSP stream can be supplied instead of 0.

Why this is important

The system processes the video frame by frame, allowing it to continuously detect and track vehicles.

4. Vehicle Detection

The detector identifies vehicles such as:

- Cars

- Buses

- Trucks

- Motorcycles

For accurate real-world detection, a trained object detector such as YOLO can be integrated with OpenCV.

Conceptually:

Input Frame

↓

Vehicle Detector

↓

Bounding Boxes

↓

Vehicle Class + Confidence

Each detected vehicle can be represented as:

(x, y, width, height)

along with its class and confidence score.

Confidence Filtering

Only detections above a chosen confidence threshold are accepted.

if confidence > 0.5:

# accept detection

This helps eliminate weak or incorrect detections.

5. Tracking Vehicles

Detection alone cannot tell whether a vehicle in the next frame is the same vehicle.

A tracker assigns a unique ID:

Frame 1 → Car ID 1

Frame 2 → Car ID 1

Frame 3 → Car ID 1

Frame 4 → Car ID 1

A simple OpenCV tracker such as CSRT can be used for individual objects, while multi-object tracking methods can be used for larger traffic scenes.

The tracking stage provides:

- Vehicle ID

- Position

- Movement direction

- Previous position

- Current position

6. Vehicle Counting

A virtual counting line can be placed across the road.

Traffic Direction

↓

🚗      🚙      🚌

↓

══════════════════════════

COUNTING LINE

══════════════════════════

When a tracked vehicle crosses the line, the count is increased.

Example:

if previous_y < line_y and current_y >= line_y:

vehicle_count += 1

The system should store the vehicle ID so that the same vehicle is not counted repeatedly.

Vehicle ID 5

↓

Crosses line

↓

Count = Count + 1

↓

ID 5 marked as counted

7. Visualization Using OpenCV Drawing Functions

OpenCV drawing functions make the system understandable to the operator.

Bounding Box

cv2.rectangle(

frame,

(x, y),

(x + w, y + h),

(0, 255, 0),

2

)

Vehicle ID

cv2.putText(

frame,

f"ID: {vehicle_id}",

(x, y - 10),

cv2.FONT_HERSHEY_SIMPLEX,

0.6,

(255, 255, 255),

2

)

Counting Line

cv2.line(

frame,

(0, line_y),

(frame.shape[1], line_y),

(0, 0, 255),

2

)

Display Statistics

cv2.putText(

frame,

f"Vehicles: {vehicle_count}",

(20, 40),

cv2.FONT_HERSHEY_SIMPLEX,

1,

(0, 255, 0),

2

)

The final video can therefore look conceptually like:

┌─────────────────────────────────────┐

│ Vehicles Detected: 12               │

│                                     │

│       ┌─────────┐                   │

│       │ Car     │                   │

│       │ ID: 15  │                   │

│       └─────────┘                   │

│              ↓                      │

│ ───────────────────────────────     │

│       COUNTING LINE                 │

│                                     │

└─────────────────────────────────────┘

8. Traffic Statistics

The system can continuously calculate:

- Total vehicles

- Cars

- Buses

- Trucks

- Motorcycles

- Vehicles per minute

- Direction of movement

For example:

╔════════════════════════╗

║ TRAFFIC STATISTICS     ║

╠════════════════════════╣

║ Cars       : 42        ║

║ Buses      :  5        ║

║ Trucks     :  8        ║

║ Motorcycles: 21        ║

║ Total      : 76        ║

╚════════════════════════╝

This information can be displayed directly on the video.

9. How Accuracy is Maintained

1. Confidence Threshold

Ignore low-confidence detections.

2. Unique Tracking IDs

Each vehicle receives an ID, preventing repeated counting.

3. Counting Line

Count only when the tracked vehicle actually crosses a predefined line.

4. Multiple-Frame Confirmation

Do not immediately accept a vehicle based on a single frame.

Detection in Frame 1

↓

Detection in Frame 2

↓

Detection in Frame 3

↓

Confirmed Vehicle

This reduces false detections caused by noise.

5. Suitable Region of Interest

Process only the road region rather than the entire CCTV image.

┌─────────────────────────────┐

│ Sky / Buildings             │

│      IGNORE                 │

├─────────────────────────────┤

│                             │

│       ROAD ROI              │

│    PROCESS THIS AREA        │

│                             │

└─────────────────────────────┘

This improves both accuracy and speed.

10. How Real-Time Performance is Maintained

Continuous traffic monitoring requires low latency.

Techniques

Frame resizing

Process a smaller frame such as:

frame = cv2.resize(frame, (640, 480))

This reduces computational workload.

ROI processing

Analyze only the road region.

Efficient detector

Use a lightweight detection model when hardware resources are limited.

Tracking between detections

Instead of performing expensive detection unnecessarily on every frame, tracking can maintain vehicle positions between detection updates.

Efficient OpenCV operations

Use optimized OpenCV functions for resizing, drawing, color conversion, and tracking.

11. Complete System Flow

LIVE CCTV

↓

Capture Frame

↓

Resize

↓

Define ROI

↓

Vehicle Detection

↓

Confidence Filtering

↓

Assign / Update ID

↓

Track Vehicle

↓

┌──────────┴──────────┐

↓                     ↓

Crosses Line?          No Crossing

↓                     ↓

YES                 Continue

↓

Increase Count

↓

Update Statistics

↓

Draw Boxes + IDs + Line

↓

Display Live Video

↓

└──────→ Next Frame

12. Evaluation

| Requirement | Proposed Solution |
| --- | --- |
| Live video | cv2.VideoCapture() |
| Vehicle detection | YOLO + OpenCV |
| Tracking | Multi-object tracking |
| Vehicle counting | Virtual counting line |
| Unique vehicles | Tracking IDs |
| Visualization | rectangle(), line(), putText() |
| Accuracy | Confidence + multi-frame confirmation |
| Speed | Resizing + ROI + efficient tracking |
| Statistics | Real-time counters |

Final Conclusion

The proposed smart traffic monitoring system combines OpenCV video handling, vehicle detection, object tracking, line-crossing counting, and visualization.

The system first captures live CCTV frames, detects vehicles, assigns tracking IDs, and follows each vehicle across consecutive frames. When a vehicle crosses a predefined counting line, its ID is checked and the vehicle is counted only once. Bounding boxes, IDs, counting lines, and traffic statistics are displayed using OpenCV drawing functions.

For accuracy, confidence filtering, ROI selection, multi-frame confirmation, and unique tracking IDs reduce false detections and duplicate counting. For real-time performance, frame resizing, ROI processing, lightweight detection, and efficient tracking reduce computational cost.

Therefore, this design provides a practical balance between accurate vehicle counting and low-latency continuous traffic monitoring.

7. Smart Attendance System Using Face Recognition

1. Objective

The system automatically marks attendance by:

- Capturing faces from a webcam.

- Detecting faces in each frame.

- Recognizing registered users.

- Marking attendance only when the identity is reliable.

- Avoiding duplicate and false attendance.

2. Proposed OpenCV Pipeline

Webcam

↓

Capture Video Frame

↓

Preprocessing

↓

Face Detection

↓

Face Alignment / Cropping

↓

Feature Extraction

↓

Face Recognition

↓

Confidence / Distance Check

↓

Multi-Frame Verification

↓

Check Already Marked?

↓

Mark Attendance

↓

Display Name + Time

3. Stage 1 – Capture Face from Webcam

OpenCV's VideoCapture() can capture live frames.

import cv2

cap = cv2.VideoCapture(0)

while True:

ret, frame = cap.read()

if not ret:

break

cv2.imshow("Webcam", frame)

if cv2.waitKey(1) & 0xFF == ord('q'):

break

cap.release()

cv2.destroyAllWindows()

Why?

The webcam provides continuous images from which faces can be detected and recognized.

4. Stage 2 – Preprocessing

The captured frame can be converted to grayscale and optionally resized.

gray = cv2.cvtColor(frame, cv2.COLOR_BGR2GRAY)

A small amount of smoothing can also be applied:

gray = cv2.GaussianBlur(gray, (3, 3), 0)

Purpose

Preprocessing:

- Reduces unnecessary information.

- Reduces small image noise.

- Makes face detection more efficient.

5. Stage 3 – Face Detection

The next step is to locate faces in the frame.

OpenCV provides Haar Cascade classifiers for basic face detection.

face_cascade = cv2.CascadeClassifier(

cv2.data.haarcascades +

"haarcascade_frontalface_default.xml"

)

faces = face_cascade.detectMultiScale(

gray,

scaleFactor=1.1,

minNeighbors=5,

minSize=(60, 60)

)

The detector returns the coordinates:

(x, y, width, height)

Then draw a rectangle:

for (x, y, w, h) in faces:

cv2.rectangle(

frame,

(x, y),

(x+w, y+h),

(0, 255, 0),

2

)

Why?

Face detection answers:

"Where is the face?"

It does not yet answer:

"Whose face is this?"

6. Stage 4 – Registered User Database

Before attendance can be taken, registered users must have reference face data.

For example:

Registered Users

Student 1 → Face Features

Student 2 → Face Features

Student 3 → Face Features

Instead of comparing raw images directly, the system should store face embeddings/features.

Conceptually:

Registered Face

↓

Feature Extraction

↓

Face Embedding

↓

Database

7. Stage 5 – Feature Extraction

The detected face is converted into a numerical representation called a feature vector/embedding.

Face Image

↓

Feature Extraction

↓

[0.21, 0.73, 0.15, ...]

A modern face-recognition model can produce an embedding that captures distinguishing facial characteristics.

The same process is performed for the live webcam face.

8. Stage 6 – Face Recognition

The live face embedding is compared with registered embeddings.

For example, using a distance measure:

A smaller distance means the two face representations are more similar.

Live Face

↓

Embedding

↓

Compare with registered embeddings

↓

Smallest valid distance

↓

Identity

9. Stage 7 – Avoiding False Recognition

This is one of the most important parts of the system.

The system should not automatically accept the closest face.

1. Recognition Threshold

Set a maximum acceptable distance.

Distance < Threshold

↓

Accept Match

Distance ≥ Threshold

↓

Unknown Person

For example, if the system uses a distance threshold of 0.5:

Distance = 0.32 → Accept

Distance = 0.72 → Reject

The exact threshold must be determined experimentally for the chosen recognition model and dataset.

2. Multi-Frame Verification

A single frame may produce an incorrect recognition.

Therefore:

Frame 1 → Student A

Frame 2 → Student A

Frame 3 → Student A

Frame 4 → Student A

↓

Confirmed Identity

Only after consistent recognition across several frames should attendance be marked.

This greatly reduces accidental recognition.

3. Unknown Person Rejection

If no registered user satisfies the recognition threshold:

Live Face

↓

Compare

↓

No valid match

↓

"Unknown"

↓

No Attendance

This prevents an unregistered person from being assigned to the nearest registered identity.

4. Face Quality Check

The system can reject poor-quality frames caused by:

- Very small faces

- Excessive blur

- Extreme pose

- Poor lighting

For example:

Poor-quality face

↓

Reject frame

↓

Wait for better frame

10. Stage 8 – Attendance Marking

Once the identity has been confirmed, attendance can be stored.

Example:

| Name | Date | Time | Status |
| --- | --- | --- | --- |
| Student A | 24-08-2026 | 10:05 AM | Present |
| Student B | 24-08-2026 | 10:07 AM | Present |

The system should maintain a set of IDs already marked for that session:

marked_users = set()

if identity not in marked_users:

mark_attendance(identity)

marked_users.add(identity)

Why?

If a student remains in front of the webcam for 30 seconds, the system should not mark attendance hundreds of times.

11. Display Results

OpenCV can display the recognized name:

cv2.putText(

frame,

"Student A",

(x, y - 10),

cv2.FONT_HERSHEY_SIMPLEX,

0.8,

(0, 255, 0),

2

)

The screen could look like:

┌──────────────────────────────┐

│                              │

│       ┌────────────┐         │

│       │            │         │

│       │    FACE    │         │

│       │            │         │

│       └────────────┘         │

│        Student A             │

│        PRESENT ✓             │

│                              │

└──────────────────────────────┘

12. Complete System Flow

WEBCAM

↓

Capture Video Frame

↓

Preprocessing

↓

Face Detection

↓

Face Quality Check

↓

Feature Extraction

↓

Generate Face Embedding

↓

Compare with Registered Data

↓

┌────────┴────────┐

↓                 ↓

Valid Match       No Valid Match

↓                 ↓

Multi-frame Check       UNKNOWN

↓                 ↓

Confirmed?          No Attendance

↓

YES

↓

Already Marked?

↓          ↓

YES         NO

↓          ↓

Ignore    Mark Present

↓

Save Date + Time

13. How the System Avoids False Recognition

| Problem | Solution |
| --- | --- |
| Similar-looking faces | Strict recognition threshold |
| Single-frame error | Multi-frame verification |
| Unknown person | Reject if distance exceeds threshold |
| Blurry image | Face-quality check |
| Poor lighting | Preprocessing / controlled illumination |
| Duplicate attendance | Maintain marked-user list |
| Wrong face detection | Minimum face-size and quality checks |

14. Final Evaluation

The proposed system uses:

OpenCV → Webcam → Face Detection → Feature Extraction → Recognition → Verification → Attendance

The most important design decision is not to mark attendance immediately after the first recognition result. Instead, the system verifies the identity using a suitable threshold and consistent recognition across multiple frames.

Conclusion

A complete OpenCV-based smart attendance system can automatically capture faces, detect them, extract facial features, compare them with registered users, and record attendance. Confidence/distance thresholds, unknown-person rejection, multi-frame verification, face-quality checks, and duplicate prevention reduce false recognition and incorrect attendance.

Therefore, the system provides a practical combination of automatic attendance, recognition reliability, and real-time webcam processing.

13. Multi-Face Tracking System

1. Objective

The system must:

- Detect multiple faces in a video.

- Track each face across consecutive frames.

- Maintain the correct identity of each person.

- Handle people entering, leaving, or crossing each other.

2. Proposed Pipeline

Webcam / CCTV Video

↓

Frame Capture

↓

Preprocessing

↓

Face Detection

↓

Face Feature Extraction

↓

Assign Unique ID

↓

Tracking

↓

Match Current Faces

with Previous Frame

↓

Identity Maintenance

↓

Draw Bounding Box + ID

↓

Next Frame

3. Stage 1: Video Capture

OpenCV captures frames continuously.

import cv2

cap = cv2.VideoCapture(0)

while True:

ret, frame = cap.read()

if not ret:

break

cv2.imshow("Multi Face Tracking", frame)

if cv2.waitKey(1) & 0xFF == ord('q'):

break

cap.release()

cv2.destroyAllWindows()

Each frame is processed individually.

4. Stage 2: Face Detection

A face detector identifies all faces in the current frame.

For example, OpenCV Haar Cascade:

face_cascade = cv2.CascadeClassifier(

cv2.data.haarcascades +

"haarcascade_frontalface_default.xml"

)

gray = cv2.cvtColor(frame, cv2.COLOR_BGR2GRAY)

faces = face_cascade.detectMultiScale(

gray,

scaleFactor=1.1,

minNeighbors=5,

minSize=(60, 60)

)

If three people are visible:

Frame

┌────────────────────────────┐

│   Face 1    Face 2         │

│      🙂         🙂          │

│                            │

│             Face 3         │

│                🙂          │

└────────────────────────────┘

The detector returns the bounding box of each face.

5. Stage 3: Assign Unique IDs

Each detected face receives a unique tracking ID.

Person 1 → ID 1

Person 2 → ID 2

Person 3 → ID 3

The important point is that the ID should remain the same as the person moves.

Example:

Frame 1:

🙂 ID 1       🙂 ID 2

↓ movement

Frame 2:

🙂 ID 1       🙂 ID 2

↓ movement

Frame 3:

🙂 ID 1       🙂 ID 2

6. Stage 4: Tracking Strategy

A good strategy is detection + tracking rather than detecting and assigning identities independently in every frame.

Why?

If we only detect faces:

Frame 1 → Face A, Face B

Frame 2 → Face A, Face B

The system does not automatically know which detection corresponds to which person.

Tracking solves this by comparing the current position with the previous position.

7. Centroid-Based Tracking

For a simple system, calculate the center of each face:

Example:

Face 1

(x,y,w,h)

↓

Centroid (Cx,Cy)

The current centroid is compared with the previous centroid.

If the distance is small:

the system assumes it is the same person.

8. Better Tracking for Multiple Faces

For a practical multi-face system, a tracker such as SORT or another multi-object tracker can be used.

A common approach is:

Face Detection

↓

Bounding Boxes

↓

Tracker

↓

Prediction

↓

Association

↓

Stable IDs

The tracker predicts where an existing face should appear in the next frame and associates new detections with those predictions.

This is more reliable than simply comparing positions.

9. Maintaining Identity

Position alone may not always be enough.

For example, two people may cross:

Before crossing:

🙂 ID 1          🙂 ID 2

↓ crossing

🙂 ID 2          🙂 ID 1

A position-only tracker may accidentally swap their identities.

To reduce this problem, combine:

1. Position

Where is the face?

2. Motion

Where is the face moving?

3. Appearance/Face Features

Does the current face look like the previously tracked face?

Position

+

Motion

+

Face Appearance

↓

Identity Matching

For identity-critical applications, face embeddings can provide a stronger identity cue than tracking alone.

10. Handling People Entering and Leaving

New person enters

New Face Detected

↓

No matching existing ID

↓

Assign New ID

↓

ID 4

Person leaves

If an ID is not detected for several frames:

ID 2 missing

↓

Wait for a few frames

↓

Still missing

↓

Remove ID 2

The waiting period prevents temporary occlusion from immediately deleting a person.

11. Handling Occlusion

Occlusion occurs when one person's face temporarily blocks another.

Example:

ID 1

↓

ID 2

ID 1 may temporarily disappear.

A good tracker keeps the ID alive for a short period using motion prediction.

When the face becomes visible again:

Predicted ID 1

+

New Detection

↓

Same ID 1

This helps maintain identity through short occlusions.

12. Visualization

OpenCV can display the tracking result:

cv2.rectangle(

frame,

(x, y),

(x+w, y+h),

(0, 255, 0),

2

)

cv2.putText(

frame,

f"ID: {track_id}",

(x, y - 10),

cv2.FONT_HERSHEY_SIMPLEX,

0.7,

(0, 255, 0),

2

)

13. Why Detection Alone is Not Enough

If we detect faces independently in every frame:

Frame 1 → Face A, Face B

Frame 2 → Face A, Face B

Frame 3 → Face A, Face B

There is no guarantee that the system maintains the same identity.

Tracking adds temporal information:

Frame 1 → ID 1, ID 2

↓

Frame 2 → ID 1, ID 2

↓

Frame 3 → ID 1, ID 2

Therefore, tracking maintains continuity across time.

14. Accuracy Improvements

| Problem | Solution |
| --- | --- |
| False face detections | Confidence/quality threshold |
| Temporary disappearance | Track persistence |
| People crossing | Position + motion + appearance |
| Occlusion | Motion prediction |
| New person | Assign new ID |
| Person leaves | Remove after timeout |
| Identity switching | Face-feature verification |

15. Real-Time Considerations

For real-time performance:

- Resize frames to a suitable resolution.

- Use a lightweight face detector.

- Detect periodically and track between detections.

- Process only the required region of interest.

- Avoid expensive face recognition on every frame.

For example:

Frame 1 → Detect + Track

Frame 2 → Track

Frame 3 → Track

Frame 4 → Detect + Track

Frame 5 → Track

This reduces computation while maintaining tracking quality.

16. Final Pipeline

VIDEO

↓

Frame Acquisition

↓

Preprocessing

↓

Multiple Face Detection

↓

┌────────┴─────────┐

↓                  ↓

Existing Face       New Face

↓                  ↓

Track / Predict      New ID

↓                  ↓

└────────┬─────────┘

↓

Appearance Matching

↓

Identity Verification

↓

Maintain Stable IDs

↓

Bounding Box + ID Display

↓

Next Frame

Conclusion

A reliable multi-face tracking system should combine face detection with multi-object tracking. Detection finds faces, while tracking maintains their identities across frames.

For a simple system, centroid/position-based tracking can work when people are well separated. For more challenging scenes, prediction + data association + appearance/face features is preferable because it reduces identity switching during movement and occlusion.

Therefore, the best practical approach is:

This provides stable identities, better handling of occlusion and crossings, and efficient real-time performance.